SOP-01 — Localized / Single-Circuit Outage

A small, contained outage — one circuit, transformer, or neighborhood — with no widespread cause.

| SOP # | SOP-01 | Applies to | Dispatchers and distribution field crews |
| --- | --- | --- | --- |
| Owner | Outage Operations | Review | Annual |

1. When this SOP applies

- Outage affecting a single circuit, transformer, or street — generally fewer than 1,000 customers.

- No evidence of widespread weather damage or transmission-level cause.

- Outage Management System (OMS) shows one or a few related events.

2. Immediate actions

- Confirm the outage in the OMS and verify the affected scope (meters/customers).

- Check for any downed-line or public-safety reports first — if present, switch to SOP-03.

- Dispatch one field crew to locate the fault.

- Publish an initial Estimated Restoration Time (ERT) to customers.

3. Roles

- Dispatcher: Confirms scope, assigns the crew, owns customer ERT.

- Field crew: Locates, isolates, and repairs the fault.

4. Procedure

- Locate the fault using OMS predictions and field patrol.

- Isolate the faulted section with the nearest switching device.

- Repair or replace the failed component (fuse, transformer, conductor, connector).

- Restore and confirm all customers are back via OMS / AMI pings.

- Close the event in OMS and record cause code.

5. Safety

- Follow standard distribution PPE and minimum approach distances.

- Treat any unexpected open point as potentially energized.

6. Communications

- Update the customer ERT through the app/IVR as the crew refines it.

- No external/media communication required for routine localized outages.

7. Restoration & closeout

- Confirm zero customers remaining in OMS.

- Log cause, equipment, and restoration time for reliability reporting.

8. Escalation thresholds

- If the count grows beyond ~1,000 customers or multiple circuits are involved, escalate to SOP-02 (Major Storm / Widespread).

- If restoration will exceed 4 hours, notify the duty supervisor and refresh customer ERT.

- If a critical-care customer is affected, also apply SOP-05.