SOP-03 — Downed Conductor / Public-Safety Hazard

A downed or low-hanging line, or any energized-equipment hazard to the public — safety first, regardless of customer count.

| SOP # | SOP-03 | Applies to | Dispatchers, first-available crews, and public-safety coordination |
| --- | --- | --- | --- |
| Owner | Outage Operations | Review | Annual |

1. When this SOP applies

- Report of a downed, sparking, or low-hanging power line.

- Energized equipment posing a public hazard (e.g., pad-mount, vault, vehicle into pole).

2. Immediate actions

SAFETY FIRST: TREAT EVERY DOWNED LINE AS ENERGIZED AND DEADLY — never assume it is de-energized.

- Dispatch the first available crew immediately, ahead of routine restoration.

- Notify 911 / local first responders and request scene control.

- Do NOT leave the hazard unattended until a Contoso crew or first responder is guarding it.

3. Roles

- Dispatcher: Prioritizes the hazard, coordinates with 911, tracks who is guarding the scene.

- First crew on scene: Establishes a safety perimeter and confirms field conditions.

4. Procedure

- Establish a safety perimeter; keep the public well clear of the line and anything it touches.

- Confirm the circuit is de-energized and verify before anyone approaches the conductor.

- Coordinate with police/fire for traffic and crowd control.

- Make the area safe, then repair and restore.

- Document the hazard, any contact/injury, and the time secured.

5. Safety

- Assume energized until tested and grounded.

- Maintain minimum approach distance; do not touch anything in contact with the line.

- Vehicles in contact with lines: occupants stay inside unless fire forces evacuation.

6. Communications

- Coordinate messaging with first responders on scene.

- Escalate immediately if there is any public contact or injury.

7. Restoration & closeout

- Confirm the hazard is cleared and the line is repaired/secured.

- File a safety report for any contact, injury, or near-miss.

8. Escalation thresholds

- Any public contact, injury, or fatality: notify Safety and senior leadership immediately and preserve the scene.

- If the hazard occurs during a major storm, this SOP takes priority over SOP-02 restoration at that location.